# REPORT TITLE: A REVIEW OF SPI and I2C

# UNIT CODE – EMT 3104: MECHATRONIC SYSTEM PROGRAMING I

# STUDENT NAME: NJERI NEWTON KARIUKI

# STUDENT NUMBER: E022-01-0794/2019

# SUBMISSION DATE: 2nd August, 2021

Contents

Introduction	3

Serial Peripheral Interface (SPI)	3

Set up of SPI	3

Atmega32 SPI Communication	4

SPI Control Register: SPCR	4

SPI Status Register: SPSR	5

SPI Data Register: SPDR	6

Inter Integrated Circuit (I2C)	6

How an I2C works	6

Atmega32 I2C Communication	7

Atmega32 I2C Module	8

Registers in atmega32 I2C Module	9

TWBR: TWI Bit Rate Register	9

TWCR: TWI Control Register	9

TWSR: TWI Status Register	10

TWDR: TWI Data Register	10

TWAR: TWI Address Register	11

References	12

# Introduction

SPI (Serial Peripheral interface) and I2C (Inter Integrated Circuit) are modern serial protocols. SPI is faster and better suited for small devices when compared to I2C, which is slower but can be extended up to a network of 128 devices using two wires and ground.

# Serial Peripheral Interface (SPI)

As mentioned in the introduction section SPIs are suitable for use where a lot of data need to be moved faster between few devices. The reason for its high speed is that, it is a synchronous/clocked protocol.

An SPI has four wires, transmitting line, receiving line, a clock line that synchronizes the communicating devices and a data framing line.

Data framing - knowing where one byte starts and where the previous one ends.

SPI devices also have 8-bit shift registers to send and receive data. Whenever a master needs to send data, it places the data on this register and generates a required clock. Whenever a master wants to read data, the slave places data on the shift register and the master generates a clock.

## Set up of SPI

Figure 1 SPI Set up

Master device – Responsible for generating the clock signal (SCK) that all of the slave devices receive. Both master and slave devices use the clock signal to synchronize their voltages on the data lines MOSI and MISO i.e. Master-out-slave-in and Master-in-slave-out lines respectively. The master device sets the voltage level on the MOSI line while the slave device sets the voltage level on the MISO line.

There is one slave-select/chip-select line per slave device that controls which slave device is currently active.

It is important to note that on every clock tick both the master and the slave devices receive and transmit a bit of data on their respective lines. This implies that both devices are sending and receiving all the time.

As stated earlier SPI are limited to a small network of devices, this is as a result of the requirement of a wire per device.

## Atmega32 SPI Communication

Atmega32 has an inbuilt SPI module. It can act as a master and slave SPI device. SPI communication pins in atmega32 are:

- MISO (Master in Slave out) – The master receives data and the slave transmits data through this pin.

- MOSI (Master out Slave in) – The master transmits data and the slave receives data through this pin.

- SCK (Shift Clock) – The master generates this clock for communication with the slave.

- SS (Slave Select) – Master can select slave device using this pin.

| SPI pins | Pin No. in Atmega32 | Pin Direction in Master Mode | Pin Direction in Slave Mode |
| --- | --- | --- | --- |
| MISO | 7(PB6) | Input | Output |
| MOSI | 6(PB5) | Output | Input |
| SCK | 8(PB7) | Output | Input |
| SS | 5(PB4) | Output | Input |

AVR atmega32 uses three registers to configure SPI communication. These registers are:

- SPI Control Register

- SPI Status Register

- SPI Data Register

### SPI Control Register: SPCR

| SPIE | SPE | DORD | MSTR | CPOL | CPHA | SPR1 | SPR0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

7		   6	           5		     4		3	       2                     1	          0

Bit 7 – SPIE: SPI Interrupt Enable bit

1 = Enable SPI interrupt

0 = Disable SPI interrupt

Bit 6 – SPE SPI Enable bit

1 = Enable SPI

0 = Disable SPI

Bit 5 – DORD: Data Order bit

1 = LSB (Least Significant Bit) transmitted first

0 = MSB (Most Significant Bit) transmitted first

Bit 4 – MSTR: Master Slave Select bit

1 = Master mode

0 = Slave mode

Bit 3 – CPOL: Clock Polarity Select bit

1 = Clock start from logical one

0 = Clock start from logical zero

Bit 2 – CPHA: Clock Phase Select bit

1 = Data sample on trailing clock edge

0 = Data sample on the leading clock edge

Bit 1:0 – SPR1: SPR0 - SPI Clock Rate Select bit

Clock rate is set as shown in the table below:

| SPI2X | SPR1 | SPR0 | SCK Frequency |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Fosc/4 |
| 0 | 0 | 1 | Fosc/16 |
| 0 | 1 | 0 | Fosc/64 |
| 0 | 1 | 1 | Fosc/128 |
| 1 | 0 | 0 | Fosc/2 |
| 1 | 0 | 1 | Fosc/8 |
| 1 | 1 | 0 | Fosc/32 |
| 1 | 1 | 1 | Fosc/64 |

### SPI Status Register: SPSR

| SPIF | WCOL |  |  |  |  |  | SPI2X |
| --- | --- | --- | --- | --- | --- | --- | --- |

7	   6	            5		     4		 3	        2		 1	        0

Bit 7 – SPIF: SPI Interrupt Flag bit

This flag gets set when the serial transfer is complete or when the SS pin is driven low in master mode. It can generate an interrupt when SPIE bit in SPCR and a global interrupt are enabled.

Bit 6 – WCOL: Write Collision Flag bit

This bit is set when SPI data register writes occurs during previous data transfer.

Bit 0 – SPI2X: Double SPI Speed bit

When set, SPI speed gets doubled. As shown in Table showing how clock rate is selected.

### SPI Data Register: SPDR

7	   6	            5		     4		 3	        2		 1	        0

This register is used to transfer data between the register file and SPI Shift Register. Writing on this register initiates data transmission.

# Inter Integrated Circuit (I2C)

I2C is a synchronous protocol for interfacing microcontrollers with small sensors and even devices that may require speed. It is also called Two Wire Interface (TWI), since it uses only two wires for communication. As it has been depicted earlier, it has the advantage of communicating with a large network of devices, up to 128 for a standard I2C, in addition it does so with much less wires i.e. two wires and a ground. The trade-off is that it is not capable of the speed of SPI and also it is more complex.

## How an I2C works

I2C uses two wires, SCL and SDA, initializations for Serial Clock and Serial Data, respectively. Since it is a synchronized protocol, it has a clock master that sends the timing signal that controls the communication.

There are four transmission modes in I2C in which the I2C device works.

- When the device is Master it works in MT and MR transmission modes.

- And when the device is Slave it works in ST and SR transmission modes.

These modes are well illustrated in the table below

| Transmission Mode | Operation |
| --- | --- |
| Master Transmitter (MT) | Master device writes data to SDA |
| Master Receiver (MR) | Master device reads data from SDA |
| Slave Transmitter(ST) | Slave device writes data to SDA |
| Slave Receiver (SR) | Slave device reads data from SDA |

There is a very strict convention about phase and polarity with two exceptions, the I2C data line can change between high and low states only while the clock line is low, and data is to be read only while the clock line is high. The two exceptions are made for start and stop signals respectively.

Start signal – a falling edge while the clock signal is held high

Stop signal – a rising edge while the clock signal is held high

Figure 2 I2C Data Timing

I2C conversation starts with pulling SDA line low before start of clock, this is the start signal. Next, the master starts the clock with a low pulse. Data is then set up on the low clock phase and read out on the higher clock phase until the transaction is over. The clock is stopped, and the master sends the stop signal by releasing the SDA.

In order to allow a number of devices to use the same two lines, each slave needs to know when it’s being communicated to, which is done by an addressing scheme. Each device has a specific seven-bit address through which communication between the master and the device must start with. The eighth bit is reserved for a read/write direction bit that determines whether the master or slave is going to send data over the data line next.

A one/high-level directional bit means that the master expects to read data off the line while a zero/low-level bit means that the master is going write the data on the SDA.

It is important to know that, the device that is receiving the data has the responsibility of acknowledging that it has received the byte. It does so by the help of a ninth bit sent after every 8-bit byte. A low (ACK) bit indicates that a byte is acknowledged and signals that the sender should continue sending. A high (NACK) bit indicates that there was an error or that the transmitter should stop sending or that the whole communication is about to end, depending on the circumstances.

## Atmega32 I2C Communication

I2C works in two modes namely,

- Master Mode

- Slave Mode

AVR atmega32 has a TWI module made up of a number of submodules as shown in the figure below.

Figure 3 AVR Atmega32 I2C Module

### Atmega32 I2C Module

SDA and SCL pins

- They are used to interface the TWI external based peripherals and the microcontroller.

- The output drivers contain a slew-rate limiter. The input stages contain a spike suppression unit which removes spikes shorter than 50 ns.

Bus Interface unit

- Contains Start/Stop control that is responsible for the generation and detection of START, REPEATED START, and STOP conditions.

- TWDR add/data shift register contain data to be transmitted and received.

- ACK bit receives ACK/NACK in transmitter mode and it is generated through software in receiving mode.

- The Spike suppression unit takes care of spikes while arbitration detection continuously monitors bus status and informs the control unit about it.

Addressing Match Unit

In slave mode, the Address match unit receives the incoming 7-bit address and compares with the address in TWAR (Two Wire Address Register) register to check whether it matches or not and upon match occur it intimate to control unit to take necessary action. It also considers the general call address if the TWGCE bit in TWAR is enabled

Control Unit

It contains the TWI control register (TWCR) and TWI status register (TWSR). It also controls the overall process of attention for necessary events, identifying events when occur, TWINT interrupts assertion, and update TWSR

Bit Rate Generator Unit

It controls the SCL in master mode to generate SCL frequency. The SCL frequency is calculated as shown below,

SCL frequency = (CPU CLK frequency)/ (16+2(TWBR)*4^TWPS)

Where; TWPS = value of a prescaler bit in TWSR

### Registers in atmega32 I2C Module

### TWBR: TWI Bit Rate Register

Used in generating SCL frequency while operating in master mode

| TWBR7 | TWBR6 | TWBR5 | TWBR4 | TWBR3 | TWBR2 | TWBR1 | TWBR0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

7	   6	            5		     4		 3	        2		 1	        0

### TWCR: TWI Control Register

This register controls all I2C communication.

| TWINT | TWEA | TWSTA | TWSTO | TWWC | TWEA |  | TWIE |
| --- | --- | --- | --- | --- | --- | --- | --- |

7	   6	            5		     4		 3	        2		 1	        0

Bit 7 – TWINT: TWI interrupt bit

Set whenever TWI completes its current event. While I-bit in SREG and TWIE bit in TWCR is enabled then TWI interrupt vector is called whenever TWI interrupt occur.

TWI interrupt flag must be cleared by software by writing a logical one to it. This bit is not automatically cleared by hardware.

Bit 6 – TWEA: TWI enable acknowledgment bit

It is set in receiver mode to generate acknowledgment and cleared in transmit mode.

Bit 5 – TWSTA: TWI START condition bit

The master device sets this bit to generate START condition by monitoring free bus status to take control over the TWI bus.

Bit 4 – TWSTO: TWI STOP condition bit

The master device sets this bit to generate STOP condition to leave control over the TWI bus.

Bit 3 – TWWC: TWI write collision bit

Set when writing to the TWDR register before the current transmission is not complete i.e. TWINT is low.

Bit 2 – TWEN: TWI enable bit

Set to enables the TWI interface in the device and takes control over the I/O pins.

Bit 0 – TWIE: TWI interrupt enable bit

Used to enable TWI to interrupt routine while the I-bit of SREG is set as long as the TWINT flag is high.

### TWSR: TWI Status Register

| TWS7 | TWS6 | TWS5 | TWS4 | TWS3 |  | TWPS1 | TWPS0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

7	   6	            5		     4		 3	        2		 1	        0

Bit 7: Bit 3 – TWS7: TWS3: TWI Status bits

These bits show the status of TWI control and bus

Bit 1: Bit 0 – TWPS1: TWPS0: TWI Prescaler bits

These bits are used to set the prescaler used in bit rate formula to calculate SCL frequency. The prescaler is se as shown in the table below;

| TWPS1 | TWPS0 | Exponent | Prescaler |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 1 | 1 | 4 |
| 1 | 0 | 2 | 16 |
| 1 | 1 | 3 | 64 |

### TWDR: TWI Data Register

This register contains data to be transmitted or received. It is not writable while TWI is in process of shifting a byte.

7	   6	            5		     4		 3	        2		 1	        0

### TWAR: TWI Address Register

This register stores the TWI unit in slave mode. It is mostly used in multi-master systems

| TWA6 | TWA5 | TWA4 | TWA3 | TWA2 | TWA1 | TWA0 | TWGCE |
| --- | --- | --- | --- | --- | --- | --- | --- |

7	   6	            5		     4		 3	        2		 1	        0

Bit 7: Bit 1 – TWA6: TWA0: TWI Address bits

These bits contain the address with which it (the device in slave mode) can be called by other masters in slave mode.

Bit 0 – TWGCE: TWI General Call Enable bit

When set it enables recognition of general call over the TWI bus

# References

- Elliot Williams (2014). Make: AVR Programming

- https://www.electronicwings.com/avr-atmega/atmega1632-spi#

- https://www.electronicwings.com/avr-atmega/atmega1632-i2c#